- Connection between two computers using Serial Cable RS-485

A. Basic Understanding Concept :-

Serial communication is a method of transmitting data one bit after another over a communication link.
Imagine we want to send:

HELLO

Inside the computer, that ultimately becomes binary data:
H → 01001000
E → 01000101
L → 01001100
L → 01001100
O → 01001111

A serial link sends those bits sequentially:
0 → 1 → 0 → 0 → 1 → 0 → 0 → 0 → ...

Python
    ↓
PySerial
  ↓
COM port
  ↓
Serial interface
  ↓
Cable
  ↓
Serial interface
  ↓
COM port
  ↓
PySerial
  ↓
Python

B.  What does "COM port" mean?

-> COM5 isn't necessarily the physical connector itself.
	It's Windows' way of exposing a serial interface to software.
	For example, suppose we plug in a USB-to-RS-485 adapter.
	Windows might say:
	USB Serial Port (COM5)

Our Python program can then communicate through:
	COM5
	PySerial handles the software side.

C. Problems with Serial Data Transfer :-

Where does the data start?
	How many data bits are there?
	Is there a parity bit?
	Where does the data end?
	How fast are the bits coming?

That's why serial communication has parameters such as:

Baud rate				Data bits
	Parity					Stop bits

START | DATA        | PARITY | STOP

|01000001|               |				This is called framing.

D. RS-485 :-

RS-485 primarily defines how the electrical signals are represented on the physical communication line.
It commonly uses two signal wires:

Device A                         Device B

A / D+ ───────────────────────── A / D+

B / D- ───────────────────────── B / D-

You'll often see these wires labelled in different ways, such as:

A / B

D+ / D-

485+ / 485-

D. UART :- (Deals with the serial data representation and timing.)
UART = Universal Asynchronous Receiver/Transmitter.

UART is hardware that converts bytes into a serial bit stream for transmission, and converts an incoming serial bit stream back into bytes.

Conceptually:

Transmit:
Byte
 ↓
UART
 ↓
serial bits

Receive:
serial bits
 ↓
UART
 ↓
Byte

Suppose we are using 				8 = 8 data bits
							N = no parity
							1 = 1 stop bit

A UART frame therefore looks conceptually like:
      START       		DATA BITS            	     STOP
          ↓             	      ↓                   	       ↓

┌───┬───────────────────────────┬─────┐

│ 0    │  	b0 b1 b2 b3 b4 b5 b6 b7  	 │     1     │

└───┴───────────────────────────┴─────┘

E. Concept of LSB :-

A = 01000001

But UART sends:
1 → 0 → 0 → 0 → 0 → 0 → 1 → 0 (Right To Left)

So conceptually our frame becomes:

START	
   ↓

0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | STOP
     ↑
   data

So, Now the timing and configuration between the two 					communicating  PC’s becomes important.

F. PySerial + First Look at COM Ports :-

PySerial is a Python library that lets our Python program communicate with serial interfaces exposed by your operating system.

It doesn't magically turn a normal USB port into RS-485.
If you have:

Laptop
  ↓
USB-to-RS485 adapter

Windows may expose that adapter as: COM5, Then PySerial can communicate with COM5.

After we connect out RS-485 to pc’s we need to check its configurations, and we would do that by doing

import serial.tools.list_ports
ports = serial.tools.list_ports.comports()

for port in ports:
    print("Device :", port.device)
    print("Name   :", port.name)
    print("Desc   :", port.description
    print("HWID   :", port.hwid)

print("-" * 40)

- Although as we don’t have an COMX port so for understanding serial communication we will use loop:// to loop our data to our same program and understand serial communication

import serial

ser = serial.serial_for_url(

"loop://",

baudrate=9600,

timeout=1

)

ser.write(b"Hello")

data = ser.read(5)

print(data)

ser.close()

//Normally, when we have real hardware, we'll do something like:

ser = serial.Serial(

"COM7",

baudrate=9600,

...

)

G. Using Dummy COMx Ports to better our code :-

->At this point, our code is like

Sender.py

import serial

ser = serial.Serial(

port="COM1",

baudrate=9600,

bytesize=8,

parity="N",

stopbits=1,

timeout=1

)

print("Sender started.")

while True:

message = input("You: ")

if message.lower() == "exit":

break

message = message + "\n"

ser.write(message.encode())

ser.close()

print("Connection closed.")

Receiver.py

import serial

ser = serial.Serial(

port="COM2",

baudrate=9600,

bytesize=8,

parity="N",

stopbits=1,

timeout=1

)

print("Receiver started.")

while True:

data = ser.readline()

if data:

message = data.decode().rstrip("\r\n")

print("Received:", message)

H. Using Threads For two way communication :-

Each computer needs to perform two independent jobs:

SEND                                                                            RECEIVE
	keyboard → serial                                                      serial → screen

import serial

import threading

PORT = "COM1"

BAUDRATE = 9600

ser = serial.Serial(

port=PORT,

baudrate=BAUDRATE,

bytesize=8,

parity="N",

stopbits=1,

timeout=0.5

)

running = True

def receive_messages():

global running

while running:

try:

data = ser.readline()

if data:

message = data.decode("utf-8").rstrip("\r\n")

print(f"\nThem: {message}")

print("You: ", end="", flush=True)

except serial.SerialException:

print("\nSerial connection error.")

running = False

receiver_thread = threading.Thread(

target=receive_messages,

daemon=True

)

receiver_thread.start()

print(f"Connected to {PORT}")

print("Type messages. Type 'exit' to quit.")

while running:

try:

message = input("You: ")

if message.lower() == "exit":

running = False

break

ser.write((message + "\n").encode("utf-8"))

except KeyboardInterrupt:

running = False

break

ser.close()

print("Serial port closed.")